Deliverable A – The LIMO Instruction Set Architecture

Contents

A1.  ISA Overview & Architecture Style	2

A2 Registers	4

List of Registers and Sesotho Names	4

Design-Decision Log Answer (Question A6a)	6

A3. Instructions	7

A4.  Sesotho Assembly	8

A5 Encoding and Specification	9

A6.  Design-decision log	11

# A1.  ISA Overview & Architecture Style

The LIMO (Lesotho Instruction Machine Architecture) is a 32-bit load–store Architecture Designed for efficiency and clarity. By ensuring that only explicit  load   store instructions  access  memory, hardware complexity is kept low, making  execution  pipelines  straight forward and predictable.

| Parameter | Specification | Description |
| --- | --- | --- |
| Word Size | 32 bits | All general-purpose registers and memory addresses are 32 bits wide. |
| Addressing Mode | Byte-addressed | Every byte in memory has a unique address. |
| Endianness | Little-endian | The least significant byte (LSB) is stored at the lowest memory address. |
| Alignment | Word-aligned | All 32-bit word loads and stores must access memory at addresses divisible by 4 (address mod 4 = 0). |
| Instruction Length | Fixed 32 bits | Every instruction is exactly 4 bytes long, simplifying instruction fetch and decoding units. |

Instruction Formats (RV32I-Derived)

LIMO uses standard 32-bit instruction layouts derived directly from RISC-V (RV32I) to ensure consistent register fields and easy decoding.

R-Type (Register-Register Operations): Used for arithmetic, logic, and shift operations between two source registers.

I-Type (Immediate & Load Operations): Used for arithmetic with immediate values, loads, and jalr.

S-Type (Store Operations): Used for writing data to memory. The immediate is split to keep register location bits (rs1, rs2) aligned with R-type instructions.

B-Type (Branch Operations): Used for conditional branching. Encodes a sign-extended 13-bit byte offset (aligned to 2-byte or 4-byte boundaries, with bit 0 implicitly zero).

J-Type (Jump Operations): Used for unconditional jumps such as jal (jump and link). The 20-bit immediate is sign-extended, shifted left by 1, and added to the PC to form the jump target address.

# A2 Registers

We chose a total of 32 registers for our processor. Registers act as fast, built-in memory slots inside the CPU that store numbers while a program is running. Choosing 32 slots gives us enough room to perform calculations and other metrics without constantly saving numbers back to main memory.

- When a processor has 32 registers, the CPU needs 5 bits to point to a specific register number (25= 32).

- An instruction that uses 3 registers rs1, rs2 and rd. For instance, adding two numbers and saving the result uses 15 bits just to select the registers as each is 5 bits.

- This leaves 17 bits out of the 32 bits for the rest of the command, such as the operation type and any immediates.

If we had chosen 16 registers instead, we would only need 4 bits per register (12 bits total), leaving slightly more space for fixed numbers, but we would run out of register slots much faster. In addition it assists us in maintaining industry standards.

## List of Registers and Sesotho Names

Here is how each of our 32 registers is assigned, including their required special roles and Sesotho names:

- r0 – Letho : Hard-wired zero (always holds the value 0)

- r1 - Khutlisa: Return address (remembers where to return after calling a function)

- r2 – tshupo : Stack pointer (tracks the top of the memory stack)

- r3 – Sebali : Program counter (tracks which instruction is currently running)

- r4 – Sesupi : Global pointer (points to global variables)

- r5 – lesupa : Frame pointer (helps keep track of function memory frames)

- r6 to r7 - Thuso0 and Thuso1: Argument registers (used to pass inputs into functions and return results)

- r8 to r15 - Nakoana0 to Nakoana7: Temporary registers (used for quick scratchpad calculations like adding up grades)

- r16 to r27 - Polokelo0 to Polokelo11: Saved registers (used to keep important variables safe while functions run)

- r28 to r31 - TseHlahlamang0 to TseHlahlamang3: Extra temporary registers for additional temporary space

# A3. Instructions

Arithmetic

- Addition

- Subtraction

- Add immediate

Logic

- Shift left logic immediate

- Or

- And

- Store

- load

Conditional

- Branch if equal

- Branch if greater than

# A4.  Sesotho Assembly

| # | LIMO Mnemonic | Meaning | Operation | RV32I Equivalent | Format |
| --- | --- | --- | --- | --- | --- |
| 1 | kop | kopanya ("to join/add") | rd = rs1 + rs2 | ADD | R |
| 2 | ts | tlosa ("to subtract/remove") | rd = rs1 − rs2 | SUB | R |
| 3 | khh | Kopanya hang -hang! ("add immediately!") | rd = rs1 + imm | ADDI | I |
| 4 | le | le ("and/with") | rd = rs1 & rs2 | AND | R |
| 5 | kp | kapa ("or") | rd = rs1 ¦ rs2 | OR | R |
| 6 | ng | ngatafala ("to multiply/shift up") | rd = rs1 << rs2 | SLLI | R |
| 7 | lt | lata ("to fetch/read") | rd = Mem[rs1 + imm] | LW | I |
| 8 | blk | boloka ("to keep/store") | Mem[rs1 + imm] = rs2 | SW | S |
| 9 | hlk | Ha e lekana ("if it is equal") | if (rs1 == rs2) PC += imm | BEQ | B |
| 10 | hft | Ha e feta ("if it passes/exceeds") | if (rs1 > rs2) PC += imm | BGT | B |

Justification for apostrophes : 

 Apostrophes are not permitted inside mnemonics.

If a new mnemonic may be required to use an apostrophe, the author may use double the letter. For example, ts’ela can be tss instead of ts’.

# A5 Encoding and Specification

1. Instruction Format Diagrams

All instructions are fixed at 32 bits long. With 32 registers, every register specifier field (rs1, rs2, rd) requires 5 bits.

R-Format (Register-to-Register Operations)

| Bits [31:25]: funct7 | Bits [24:20]: rs2 | Bits [19:15]: rs1 | Bits [14:12]: funct3 | Bits [11:7]: rd | Bits [6:0]: opcode |
| --- | --- | --- | --- | --- | --- |

I-Format (Immediate Operations & Loads)

| Bits [31:20]: Imm [11:0] | Bits [19:15]: rs1 | Bits [14:12]: funct3 | Bits [11:7]: rd | Bits [6:0]: opcode |
| --- | --- | --- | --- | --- |

S-Format (Store Operations)

| Bits [31:25]: Imm [11:5] | Bits [24:20]: rs2 | Bits [19:15]: rs1 | Bits [14:12]: funct3 | Bits [11:7]: Imm [4:0] | Bits [6:0]: opcode |
| --- | --- | --- | --- | --- | --- |

B-Format (Conditional Branches)

| Bits [31]: Imm [12] | Bits [30:25]: Imm [10:5] | Bits [24:20]: rs2 | Bits [19:15]: rs1 | Bits [14:12]: funct3 | Bits [11:8]: Imm [4:1] | Bits [7]: Imm [11] | Bits [6:0]: opcode |
| --- | --- | --- | --- | --- | --- | --- | --- |

2. Opcode/Funct Assignments and Immediate Ranges

- R-Format: Opcode 0110011, funct3 varies (000 to 111), funct7 = 0000000 / 0100000.    Immediate Range: N/A.

- I-Format (Immediate Arithmetic): Opcode 0010011, funct3 varies (000 to 111). Immediate Range: 12-bit signed (-2048 to +2047).

- I-Format (Word Load): Opcode 0000011, funct3 = 010. Immediate Range: 12-bit signed offset (-2048 to +2047 bytes).

- S-Format (Word Store): Opcode 0100011, funct3 = 010. Immediate Range: 12-bit signed offset (-2048 to +2047 bytes).

- B-Format (Branches): Opcode 1100011, funct3 = 000 BEQ / 001 BNE. Immediate Range: 13-bit signed offset in multiples of 2 (-4096 to +4094 bytes).

3. Hand-Encoded Examples (Binary and Hexadecimal)

R-Format Example: kop r8, r6, r7 (ADD t0, a0, a1)

- Operation: Add contents of r6 and r7 and save result into r8, r8 = r6 + r7.

- Binary Encoding: 0000000 00111 00110 000 01000 0110011

- Hexadecimal Code: 0x00730433

I-Format Example: khh r8, r6, 15 (ADDI t0, a0, 15)

- Operation: Add constant 15 to r6 and store result into r8, r8 = r6 + 15.

- Binary Encoding: 000000001111 00110 000 01000 0010011

- Hexadecimal Code: 0x00F30433

S-Format Example: blk r7, 4(r2) (SW a1, 4(sp))

- Operation: Store word from r7 (Thuso1) into memory address r2 + 4 (Stack Pointer + 4).

- Binary Encoding: 0000000 00111 00010 010 00100 0100011

- Hexadecimal Code: 0x00712243

B-Format Example: lek r6, r7, +16 (BEQ a0, a1, +16)

- Operation: Jump relative byte offset +16 if contents of r6 equal r7.

- Binary Encoding: 0 000000 00111 00110 000 1000 0 1100011

- Hexadecimal Code: 0x00730863

# A6.  Design-decision log

- Our LIMO implements 32 architectural registers (RV32I-derived),  having 32 registers gives our processor enough space to handle calculations (like student grades) directly inside the CPU without constantly saving and loading data from memory. The downside is that each register address takes up 5 bits in an instruction (15 bits total for 3 registers), which leaves less room for constant numbers compared to a 16-register setup. In hardware, it also means using slightly larger circuits to compare register numbers when handling hazards. Every instruction in our LIMO processor is fixed at 32 bits total in length.

- Architectural: x0–x31 (x0 hardwired to zero), PC. These are visible to the ISA and must be preserved across exceptions. Microarchitectural only: IR (instruction register), IF/ID pipeline register, ID/EX, EX/MEM, MEM/WB, and any internal forwarding or temporary registers. The ISA must not specify these because they are implementation artefacts , that is, a different pipeline depth, out-of-order design, or single-cycle implementation would need different internal registers. Specifying them would freeze one microarchitecture and destroy compatibility across LIMO implementations. Only the programmer-visible state (registers, PC, memory) belongs in the ISA.

- In our LIMO pipeline, branches resolve in the EX stage, where the ALU compares rs1 and rs2 and computes the target. With a classic 5-stage pipeline (IF, ID, EX, MEM, WB), this means two instructions fetched after the branch (in IF and ID) must be flushed when the branch is taken. Cost: up to 2 flushed instructions per taken branch, plus the branch penalty cycles. Extra hardware cost is small, that is, a comparator and target adder in EX. Resolving earlier in ID would need extra forwarding/compare hardware but reduce flushes to 1; resolving in MEM would need 3 flushes but almost no extra hardware, so EX is the balanced choice.

- Full forwarding cannot eliminate the load-use hazard because the loaded value is only available after the MEM stage, while the dependent instruction needs it at the start of its EX stage — one cycle too early. Cycle diagram (I1 = lw x1, 0(x2); I2 = add x3, x1, x4).

| Cycle | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| I1 (lw) | IF | ID | EX | MEM | WB |
| L2 (add) | IF | ID | Stall EX | MEM | WB |

I2's EX would need x1 at cycle 4, but I1 writes it at end of cycle 4 (WB). Forwarding MEM→EX gives it at cycle 5, so one stall bubble is mandatory.

- A flags register would create a new architectural state and a new hazard class: every ALU instruction would implicitly write flags, so a branch depending on flags would need to wait for the last flag-writing instruction even if that instruction's result register isn't otherwise needed. This adds false dependencies and complicates forwarding (flags become a second write port and a second forwarding path). It might save an encoding bit or two on branches, but the hazard cost and extra forwarding comparators outweigh that benefit. LIMO is better without flags.

- immediate range breaks first. LIMO's I-type immediate is 12 bits (sign-extended, ±2048), S-type is 12 bits, B-type is 13 bits (±4 KiB branch reach), and J-type is 20 bits (±1 MiB jump reach). Register count (32) is generous and rarely the first limit. Branch reach becomes a problem only in very large functions. But immediate range bites almost immediately: array offsets, constants, and stack frame sizes beyond ±2048 require extra lui/addi sequences, inflating code size and instruction count long before register pressure or branch reach become critical.

7. 3 Sample LIMO Programs

Program 1: Addition(Kopanya)

# Simple addition: r8 = r6 + r7

# Inputs:  r6 = first number, r7 = second number

# Output:  r8 = sum

kop   r8, r6, r7      # r8 = r6 + r7

Program 2: store a constant(boloka palo)

# Simple immediate + store: r8 = r6 + 15, then Mem[r2+4] = r8

# Inputs:  r6 = base value, r2 = stack pointer(Tshupo)

# Output:  memory word at address (r2 + 4)

khh   r8, r6, 15      # r8 = r6 + 15   (I-format immediate add)

blk   r8, 4(r2)       # Mem[r2 + 4] = r8   (S-format store)

Program 3:  Simple average (midpoint) of two numbers

# Inputs:  r6 = a, r7 = b

# Output:  r8 = (a + b) / 2

kop   r8, r6, r7      # r8 = a + b        (R-format add)

khh   r9, r0, 1       # r9 = 1            (I-format, r0 is hardwired zero)

ng    r8, r8, r9      # r8 = r8 >> 1      (logical shift right by 1 = divide by 2)